# How to update with PortUpgrade program

Step 1: SET RS-232 DIP switch to FW

Step 2: Connect Rx150cs rs-232 port to computer and power on rx150cs

Step 3: Open portupgrade software and selecte the com port connected to the rs-232 port.

Step 4: click “connect” and wait until the button becomes green to show it has connected to the rx150cs successfully.

step 5: click “browse” and selec the firmware file for the rx150cs

Step 6: Click “upgradestart” then “forceupgrade” to begin update.

Step 7: wait unitl the window reads “upgrade success” indicating that eh upgrade is completed.